Technical Specifications Bowen S3.5 Power Swivel Skid Mounted Full Rebuild & Commissioning 2025

| ID Number: | 0506-0-003-C | Unit serial number: | #998 |
| --- | --- | --- | --- |
| Operating Hours: | Zero | Present Location: | UAE |
| Assembly Part Number: | 84550 | Inspection and testing: | Re-Build 2025 |

| Manufacturer: | Bowen | Model: | S 3.5 (PS 120) |
| --- | --- | --- | --- |
| Rated capacity: | 120T | Engine Horsepower Continuous: | New Cummins 6BTAA5.9-C205 210 Bhp@ 2200 RPM |
| Maximum Circulating Pressure: | 5,000 PSI | Maximum Speed RPM: | 150 |

| Height: | 85’’ 2.159M | Width: | 85’’ 2.159M |
| --- | --- | --- | --- |
| Length: | 168’’ 4.2672 M | Weight: | 11,500 Lbs. (5216 kg) |

| Radius of Torque Rein Telescoped: | 44 1/8” | Radius of Torque Rein Extended: Telescope | 64 1/8” |
| --- | --- | --- | --- |
| Bore thru stem : | 2-1/4” | Elevator Bail Pipe Size: | 3 ½” & 4 ½” |
| Bail Hook Clearance: | 23-1/4” | Goose Neck connection: | 3” -8 Line Pipe Thread |
| Maximum Torque: | 8,100 Ft Lbs. | Saver Sub Connection: | 3.5” IF |
| Saver Sub make up torque: | 7,500/10,000 Ft Lbs. | HP Hose Connection Forward : | 1-1/4” x 120’ |
| Max. RPM: | 150 | HP Hose Connection Reverse: | 1-1/4” x 120’ |
| HP Hydraulic Hose Connection | Quick Release | Case Drain Hose Size: | 1” x 120’ |